Seenpin V2.0 CAN-FD User Manual

- Basic Operations

Operation 1: SDO Parameter Read

Function: Read parameters (e.g., Device ID, software version, position offset) of the specified main index and subindex from the motor.

- Construct a CAN FD message in accordance with the SDO read message format, with COB_ID=0x600+Node_ID and DLC=8.

- Definition of message data segment:

·  Byte 0: Command word, fixed as 0x40;

·  Bytes 1-2: Object main index (low byte + high byte);

·  Byte 3: Object subindex;

·  Bytes 4-7: Fixed as 0x00.

- After sending the above message, the motor will reply with an SDO message with COB_ID=0x580+Node_ID and DLC=8.

- Parse the reply message:

·  Byte 0: Command word (0x4F = 1-byte data, 0x4B = 2-byte data, 0x47 = 3-byte data, 0x43 = 4-byte data, 0x80 = read failure);

·  Bytes 1-2: Main index, Byte 3: Subindex (consistent with the sent message);

·  Bytes 4-7: Returned parameter data (intercept the corresponding bytes according to the command word, little-endian format).

Example: Read the PosOffset (main index 0x2001, subindex 5) of the motor with Node_ID=2. Send a message with COB_ID=0x602 and the data segment as 0x40 01 20 05 00 00 00 00.

Operation 2: SDO Parameter Write Operation

Function: Write parameters (e.g., modify Node_ID, set heartbeat interval, configure control precision) to the specified main index and subindex of the motor.

- Determine the number of bytes of the parameter to be written and select the corresponding command word: 1 byte = 0x2F, 2 bytes = 0x2B, 3 bytes = 0x27, 4 bytes = 0x23.

- Construct an SDO write message with COB_ID=0x600+Node_ID and DLC=8.

- Definition of message data segment:：

·  Byte 0: Command word corresponding to the number of bytes;

·  Bytes 1-2: Object main index (low byte + high byte);

·  Byte 3: Object subindex;

·  Bytes 4-7: Parameter data to be written (little-endian format, fill with 0x00 if insufficient).

- After sending the message, the motor will reply with an SDO confirmation message with COB_ID=0x580+Node_ID and DLC=8.

- Parse the confirmation message: Byte 0 is 0x60 for write success, 0x80 for write failure (check parameter format/range).

Example: Modify the Node_ID of the motor with Node_ID=1 to 2. Send a message with COB_ID=0x601 and the data segment as 0x2F 01 20 01 02 00 00 00.

Operation 3: Heartbeat Interval Configuration

Function: Set the transmission cycle of the motor heartbeat message, 0x00 to disable heartbeat (default 100ms, main index 0x1017, subindex 0).

- Adopt the SDO write operation with the command word 0x2F (1-byte data), main index 0x1017 and subindex 0.

- The written data is the heartbeat interval (unit: ms, 0x00 = disabled, e.g., 0x02=2ms, 0x64=100ms).

- Example: Disable the heartbeat of the motor with Node_ID=1. Send a message with COB_ID=0x601 and the data segment as 0x2F 17 10 00 00 00 00 00.

Operation 4: PDO Reporting Configuration

Function: Enable/disable the motor PDO1/PDO4 data reporting and set the reporting cycle (main index 0x1800 + subindex 5 = PDO1, main index 0x1803 + subindex 5 = PDO4, 0x00 = disabled).

- Adopt the SDO write operation with the command word 0x2F (1-byte data) and the corresponding main index + subindex.

- The written data is the reporting cycle (unit: ms, 0x00 = disabled, e.g., 0x02=2ms).

- Example: Set the PDO4 reporting cycle of the motor with Node_ID=1 to 2ms. Send a message with COB_ID=0x601 and the data segment as 0x2F 03 18 05 02 00 00 00.

II. Motor Control Mode Operation Process

General Pre-steps (for all control modes)

- Complete the CAN FD bus connection and communication parameter configuration, and confirm the normal communication between the controller and the motor (verify by reading Node_ID via SDO).

- Configure the heartbeat interval and PDO reporting cycle as needed (e.g., it is recommended to enable PDO1/PDO4 for real-time control with a reporting cycle of 2-10ms).

- Motor power-on enable: Send an enable message via PDO1 with COB_ID=0x200+Node_ID, DLC=7 and the data segment as 00 00 00 00 00 07 00.

- After successful enabling, the motor enters the standby state and can be switched to the specified control mode.

Mode 1: Current Mode

Function: Precisely control the motor output current (unit: 0.01A), suitable for current closed-loop control scenarios.

- Complete the general pre-steps and enable the motor power-on.

- Send a PDO1 control message with COB_ID=0x200+Node_ID, DLC=7. Definition of data segment:

·  Byte 0: Mode code, fixed as 04 (Current Mode);

·  Bytes 1-2: Target current (0.01A, little-endian format, e.g., 0x64=1A, 0xC8=2A);

·  Bytes 3-4: Rise time (ms, 0x64=100ms, 0x00 = maximum speed);

·  Bytes 5-6: Control word, fixed as 0F 00.

- The motor outputs current according to the set parameters. The feedback current can be monitored in real time through the PDO1 receive message (COB_ID=0x180+Node_ID).

- After the operation, disable the motor: Send a PDO1 message with COB_ID=0x200+Node_ID, DLC=7 and the data segment as 00 00 00 00 00 04 00. The motor will power off and stop outputting current.

Example: For the motor with Node_ID=1, output 2A in current mode with a rise time of 100ms. Send a message with COB_ID=0x201 and the data segment as 04 C8 00 64 00 0F 00.

Mode 2: Position Mode

Function: Precisely control the motor to move to the specified absolute position (unit: 0.01mm), suitable for position closed-loop control scenarios.

- Complete the general pre-steps and enable the motor power-on.

- Send a PDO1 control message with COB_ID=0x200+Node_ID, DLC=7. Definition of data segment:

·  Byte 0: Mode code, fixed as 01 (Position Mode);

·  Bytes 1-2: Target position (0.01mm, little-endian format, e.g., 0x64=1mm, 0xC8=2mm);

·  Bytes 3-4: Movement time (ms, 0x64=100ms, 0x00 = maximum speed);

·  Bytes 5-6: Control word, fixed as 0F 00.

- The motor moves to the target position according to the set parameters and maintains it. The current position can be monitored in real time through the PDO1 receive message.

- After the operation, perform the motor disable step (same as Current Mode). The motor will power off and no longer maintain the position.

Example: For the motor with Node_ID=1, move to 2mm in position mode with a movement time of 100ms. Send a message with COB_ID=0x201 and the data segment as 01 C8 00 64 00 0F 00.

Mode 3: Thrust Mode

Function: Precisely control the motor output thrust (unit: N), suitable for thrust closed-loop control scenarios.

- Complete the general pre-steps and enable the motor power-on.

- Send a PDO1 control message with COB_ID=0x200+Node_ID, DLC=7. Definition of data segment:

·  Byte 0: Mode code, fixed as 02 (Thrust Mode);

·  Bytes 1-2: Target thrust (N, little-endian format, e.g., 0x64=100N, 0xC8=200N);

·  Bytes 3-4: Transition time (ms, 0x64=100ms, 0x00 = direct output);

·  Bytes 5-6: Control word, fixed as 0F 00.

- The motor outputs thrust according to the set parameters. The current thrust can be monitored in real time through the PDO1 receive message.

- After the operation, perform the motor disable step (same as Current Mode). The motor will power off and stop outputting thrust.

Example: For the motor with Node_ID=1, output 200N in thrust mode with direct output (transition time 0). Send a message with COB_ID=0x201 and the data segment as 02 C8 00 00 00 0F 00.

Mode 4: Speed Mode

Function: Precisely control the motor running speed (unit: 0.1mm/s), suitable for speed closed-loop control scenarios.

- Complete the general pre-steps and enable the motor power-on.

- Send a PDO1 control message with COB_ID=0x200+Node_ID, DLC=7. Definition of data segment:

·  Byte 0: Mode code, fixed as 03 (Speed Mode);

·  Bytes 1-2: Target speed (0.1mm/s, little-endian format, e.g., 0x64=10mm/s, 0xC8=20mm/s);

·  Bytes 3-4: Acceleration time (ms, 0x64=100ms, 0x00 = run at target speed directly);

·  Bytes 5-6: Control word, fixed as 0F 00.

- The motor runs according to the set parameters. The current speed can be monitored in real time through the PDO1 receive message.

- After the operation, perform the motor disable step (same as Current Mode). The motor will power off and stop running.

Example: For the motor with Node_ID=1, run at 20mm/s in speed mode with direct operation (acceleration time 0). Send a message with COB_ID=0x201 and the data segment as 03 C8 00 00 00 0F 00.

Mode 5: PVT Mode (Position-Velocity-Thrust)

Function: Force-position hybrid control, the motor exhibits spring-damper characteristics, suitable for scenarios requiring flexible control.

Core Formula: F = (TargetPosition - ActualPosition)*Kp + (TargetVelocity - ActualVelocity)*Kd + TargetForce

Exclusive Pre-configuration

·  Disable motor heartbeat: Execute Operation 3, write 0x00 to 0x1017 subindex 0 via SDO.

·  Enable PDO4 reporting: Execute Operation 4, write the specified cycle (e.g., 2ms) to 0x1803 subindex 5 via SDO.

·  Enter standby mode: SDO write operation, COB_ID=0x600+Node_ID, command word 0x2B, main index 0x2011, subindex 01, data 0x2000.

·  Switch to force sensor control: SDO write operation, COB_ID=0x600+Node_ID, command word 0x2B, main index 0x2020, subindex 00, data 0x0100.

·  Motor enable: SDO write operation, COB_ID=0x600+Node_ID, command word 0x2B, main index 0x2011, subindex 01, data 0x4000.

Real-time Control Steps

- Send PDO4 control messages periodically with COB_ID=0x500+Node_ID, DLC=14, and the data segment in little-endian format with the following definition:

| Data Offset | Type | Description | Unit |
| --- | --- | --- | --- |
| 0 | uint16 | Motor Control Word | Fixed as 0x4100 |
| 2 | int16 | Target Position | 0.00001m |
| 4 | int16 | Target Velocity | 0.0001m/s |
| 6 | int16 | Target Torque (Thrust Compensation) | 1N |
| 8 | uint16 | Kp (Elastic Coefficient) | 0.1N/mm |
| 10 | uint16 | Kd (Damping Coefficient) | 0.1N/(mm/s) |
| 12 | uint16 | DataIndex | Increment by 1 for each transmission |

- Monitor the motor status in real time through the PDO4 receive message (COB_ID=0x480+Node_ID), including actual position, velocity, torque, current, temperature, etc.

Post-operation Steps

After the operation, disable the motor: SDO write operation, COB_ID=0x600+Node_ID, command word 0x2B, main index 0x2011, subindex 01, data 0x4F00.

Example: For the motor with Node_ID=1 in PVT mode, target position 20mm (0x07D0), velocity 0, thrust compensation 100N (0x64), Kp=8000 (0x1F40), Kd=200 (0xC8), DataIndex=1. Send a message with COB_ID=0x501 and the data segment as 00 41 D0 07 00 00 64 00 40 1F C8 00 01 00.

Mode 6: Force-Position Hybrid Control

Function: Realize hybrid closed-loop control of position, velocity and thrust through PDO2, supporting precise gain adjustment.

·  Complete the general pre-steps and enable the motor power-on.

·  Send a PDO2 control message with COB_ID=0x300+Node_ID, DLC=12. The data segment includes position, velocity, thrust, position gain Kp, velocity gain Kd and gain precision.

·  Gain calculation: Kp = Position gain Kp/(2^KP precision), Kd = Velocity gain Kd/(2^Kd precision) (the default KP/Kd precision is 10, at this time the unit of Kp is approximately 0.01N/mm, Kd is approximately 0.01N/(mm/s)).

·  Monitor the motor status in real time through the PDO2 receive message (COB_ID=0x280+Node_ID), including motor status word, actual position, velocity and thrust.

·  After the operation, perform the motor disable step (same as Current Mode).

III. PDO Data Reception and Parsing

PDO1 Reception

COB_ID=0x180+Node_ID, DLC=16, used for real-time collection of basic motor status. Parsing of the data segment is as follows:

·  Bytes 0-1: Motor fault status (Status bit, e.g., bit1 = overvoltage, bit7 = encoder communication abnormality, etc.);

·  Bytes 2-3: Current position (×0.01mm);

·  Bytes 4-5: Current speed (×0.1mm/s);

·  Bytes 6-7: Current thrust (N);

·  Bytes 8-9: Feedback current (×0.01A);

·  Bytes 10-15: Reserved.

PDO2 Reception

COB_ID=0x280+Node_ID, DLC=8, used for status collection of force-position hybrid control:

·  Bytes 0-1: Status word;

·  Bytes 2-3: Actual position;

·  Bytes 4-5: Actual velocity;

·  Bytes 6-7: Actual thrust.

PDO4 Reception

COB_ID=0x480+Node_ID, DLC=32, exclusive status collection for PVT mode, including actual position, velocity, torque, three-phase current, voltage, temperature, error code, etc. (see the PDO4 reception format in the Term Definition section for details).

IV. Fault Handling

Fault Code and Status Bit Query

·  Quickly judge the basic motor faults (e.g., overvoltage, undervoltage, encoder abnormality, overcurrent, etc.) through the Status bit of the PDO1 receive message.

·  Read the motor error code (uint16) through Bytes 8-9 of the PDO4 receive message to locate PVT mode exclusive faults.

·  Read the motor general error code by reading 0x2010 subindex 5 (ErrCode) via SDO; read 0x6041/0x6042/0x6043/0x6044 to read the detailed fault status (e.g., excessive current, MOSFET fault, overhigh temperature).

Common Faults and Solutions

| Fault Phenomenon | Fault Cause | Solution |
| --- | --- | --- |
| Communication abnormality, no SDO/PDO reply | CAN FD bus wiring fault / baud rate mismatch / incorrect Node_ID | Check the bus wiring, reconfigure the baud rate of the controller and motor, confirm that the Node_ID is unique and correct |
| Overvoltage alarm (Status bit1) | Motor supply voltage exceeds the rated value | Check the power supply and reduce the voltage to the rated range |
| Undervoltage alarm (Status bit2) | Motor supply voltage is lower than the rated value | Check the power supply, increase the voltage to the rated range, and troubleshoot line voltage drop |
| Encoder communication abnormality (Status bit7) | Encoder wiring fault / encoder damage | Check the encoder wiring, replace the encoder if damaged, and recalibrate the encoder parameters |
| Overcurrent alarm (Status bit9) | Excessive motor load / three-phase short circuit / incorrect current parameter configuration | Reduce the motor load, troubleshoot the three-phase line short circuit, and recalibrate the current loop P/I parameters and rated current |
| Overhigh temperature alarm | Motor/driver temperature exceeds the threshold | Stop the motor operation, troubleshoot heat dissipation faults, and adjust the temperature threshold by writing to 0x200B subindex 6/7 via SDO (operate with caution) |
| No PDO reporting data | PDO reporting not enabled / incorrect cycle configuration | Re-execute the PDO reporting configuration operation and confirm writing a non-0x00 reporting cycle |
| SDO write failure (reply 0x80) | Incorrect parameter format/range / read-only parameter | Check the number of parameter bytes and value range, confirm that the parameter is writable (rw), do not modify read-only (ro) parameters |

Fault Reset

- Minor faults (e.g., overtemperature alarm, undervoltage alarm): After eliminating the fault, perform the motor disable + re-enable operation, and the fault will reset automatically.

- Severe faults (e.g., overcurrent, encoder fault, MOSFET fault): After eliminating the fault, restart the motor power supply, and then re-execute the power-on enable process.

V. Notes

·  Parameter Protection: Do not modify the factory-calibrated motor parameters (marked as "Factory-calibrated, do not modify", e.g., screw lead, current sampling resistor, motor pole pairs), otherwise it will cause abnormal motor control or even damage.

·  Mode Switching: The motor disable operation must be performed before switching the motor control mode. Direct mode switching in the enabled state is prohibited.

·  CAN FD Bus: In multi-machine communication, the Node_ID of each motor must be unique to avoid message conflict; the bus length should not be too long to ensure communication stability.

·  Safe Operation: Do not touch the moving parts during motor operation; ensure the motor is in the disabled state when modifying parameters and troubleshooting faults to prevent accidental start-up.

·  Data Format: All SDO/PDO message data is in little-endian format. Strictly follow this rule when parsing/constructing messages to avoid data errors.

·  PVT Mode: The DataIndex of PDO4 must be incremented by 1 for each transmission. Sending the same DataIndex repeatedly is prohibited, otherwise the control command will be invalid.

·  Thrust Zero Calibration: If there is a deviation in thrust detection, write 0 to 0x2002 subindex 1 (calibration) via SDO to set the current force sensing value as the zero point.

·  Position Calibration: If the feedback position differs greatly from the actual position, write 0 to 0x2001 subindex 5 (PosOffset) via SDO, then calculate the difference between the actual position and the feedback position, and write the difference to PosOffset.PosOffset = Actual Position - Feedback Position.

Appendix 1: Object Dictionary

| Main Index | Subindex | Data Type | Object Description | Content | R/W | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| 0x1017 | 0 | UINT8 | Heartbeat Interval | 100 | rw | Unit: ms, 0x00 to disable heartbeat |
| 0x1800 | 5 | UINT8 | PDO1 Data Reporting | 0 | rw | Unit: ms, 0x00 to disable data reporting |
| 0x1803 | 5 | UINT8 | PDO4 Data Reporting | 0 | rw | Unit: ms, 0x00 to disable data reporting |
| 0x2001 | 0 | UINT8 | Device Info Parameter | 6 | ro |  |
|  | 1 | UINT8 | Device ID(Node_ID) | 1 | rw | Device communication ID |
|  | 2 | UINT8 | Reboot | 0 | rw | Reserved |
|  | 3 | UINT8 |  |  |  | Reserved |
|  | 4 | UINT32 | LeadNum |  | rw | Screw lead, the telescopic length of the screw per motor revolution, unit: um, factory-calibrated, do not modify |
|  | 5 | INT16 | PosOffset |  | rw | Screw position offset (correction value), unit: 10um |
|  | 6 | UINT32 | SamplingRes |  | rw | Current sampling resistance value, unit: 0.1mΩ, factory-calibrated, do not modify |
| 0x2002 | 0 | UINT8 | push press Parameter | 4 | ro |  |
|  | 1 | INT32 | calibration |  | rw | Thrust zero calibration, write 0 to set the current force sensing value as the zero point |
|  | 2 | INT16 | DZERO |  | rw | - |
|  | 3 | INT32 | DK |  | rw | Thrust coefficient, precision 0.000001, factory-calibrated, do not modify |
|  | 4 |  |  |  |  |  |
|  | 5 |  |  |  |  |  |
|  | 6 | UINT16 | Sensor Range |  |  | Unit: N, supported by software version 213 or 221, set the upper limit of the sensor return value, 0 for no upper limit |
| 0x2003 | 0 | UINT8 | driver_info | 6 | ro |  |
|  | 1 |  |  |  |  |  |
|  | 2 | UINT16 | Driver fver |  | ro | Software version |
|  | 3 | UINT16 | Driver hver |  | ro | Hardware version |
|  | 4 | UINT32 | Baudrate |  | rw | Serial port baud rate, reserved |
|  | 5 | UINT16 | DeadTime |  | rw | Dead time, unit: ns, factory-calibrated, do not modify |
|  | 6 | UINT16 | VoutMax | 0~10000 | rw | Maximum output duty cycle, unit: 0.01%, factory-calibrated, do not modify |
| 0x2004 | 0 | UINT8 | Position Control | 4 | ro |  |
|  | 1 | UINT32 | P |  | rw | Position loop P, precision 0.00001 |
|  | 2 | UINT32 | I |  | rw | Position loop I, precision 0.00001 |
|  | 3 | UINT32 | D |  | rw | Position loop D, precision 0.00001 |
|  | 4 | UINT32 | Precision |  | rw | Position control precision (allowable error), unit: um |
| 0x2005 | 0 | UINT8 | Speed Control | 5 | ro |  |
|  | 1 | UINT32 | P |  | rw | Speed loop P, precision 0.00001 |
|  | 2 | UINT32 | I |  | rw | Speed loop I, precision 0.00001 |
|  | 3 | UINT32 | D |  | rw | Not used |
|  | 4 | UINT32 | Precision |  | rw | Speed control precision (allowable error), unit: um/s |
|  | 5 | UINT32 | DeadCurrent |  | rw | Dead current, unit: mA |
| 0x2006 | 0 | UINT8 | Iq Current Control | 3 | ro |  |
|  | 1 | UINT32 | P |  | rw | Current loop P, precision 0.00001 |
|  | 2 | UINT32 | I |  | rw | Current loop I, precision 0.00001 |
|  | 3 | UINT32 | D |  | rw | Not used |
| 0x2007 | 0 | UINT8 | Id Current Control | 3 | ro |  |
|  | 1 | UINT32 | P |  | rw | Current loop P, precision 0.00001 |
|  | 2 | UINT32 | I |  | rw | Current loop I, precision 0.00001 |
|  | 3 | UINT32 | D |  | rw | Not used |
| 0x2008 | 0 | UINT8 | Thrust Control | 8 | ro |  |
|  | 1 | UINT32 | P |  | rw | Thrust loop P, precision 0.00001 |
|  | 2 | UINT32 | I |  | rw | Not used |
|  | 3 | UINT32 | D |  | rw | Thrust loop D, precision 0.00001 |
|  | 4 | UINT32 | Precision |  | rw | Thrust control precision (allowable error), unit: 0.1N |
|  | 5 | UINT32 | TorScale |  | rw | Torque to thrust conversion ratio, precision 0.00001 |
|  | 6 | UINT32 | StackFric_K |  | rw | Static friction coefficient, precision 0.00001 |
|  | 7 | UINT32 | DOBFliter_K |  | rw | Disturbance observer filter coefficient, precision 0.00001 |
|  | 8 | UINT32 | TargetFliter_K |  | rw | Command filter coefficient, precision 0.00001 |
| 0x2009 | 0 | UINT8 | Motor parameter | 6 | ro |  |
|  | 1 | UINT8 | PolePairs |  | rw | Motor pole pairs, factory-calibrated, do not modify |
|  | 2 | UINT16 | NormalCurrent |  | rw | Rated current, unit: 10mA, factory-calibrated, do not modify |
|  | 3 | UINT16 | MaxCurrent |  | rw | Peak current, unit: 10mA, factory-calibrated, do not modify |
|  | 4 | UINT32 |  |  | rw |  |
|  | 5 | UINT32 | MaxRPM |  | rw | Maximum speed |
|  | 6 | UINT8 | MotorDirection |  | rw | Motor phase sequence direction |
| 0x2010 | 0 | UINT8 | Motor Status | 5 | ro |  |
|  | 1 | UINT16 | Statusword |  | ro | Controller status 0x20xx---Enter standby mode 0x40xx---Enter servo mode 0x41xx---Enter PVT mode 0x4Fxx---Enter servo-off mode |
|  | 2 | INT16 | Position |  | ro | Unit: 10um |
|  | 3 | INT16 | Speed |  | ro | Unit: 100um/s |
|  | 4 | INT16 | Force |  | ro | Unit: N |
|  | 5 | UINT16 | ErrCode |  | ro | Error code |
| 0x200A | 0 | UINT8 | encoder parameter | 4 | ro |  |
|  | 1 | UINT8 | Type | 0 | rw | Encoder type |
|  | 2 | UINT8 | Ncycles | 1 | rw | Encoder multi-turn |
|  | 3 | UINT8 | Bits | 16 | rw | Encoder resolution 16 bits |
|  | 4 | UINT8 | Reverse | 1 | rw | Encoder mechanical angle direction |
|  | 5 | UINT32 | LostCount |  | r | Packet loss count |
| 0x200B | 0 | UINT8 | sensor | 7 | ro |  |
|  | 1 | INT16 | BusBarVoltage |  | ro | Bus voltage |
|  | 2 | INT16 | DriverTemperature1 |  | ro | Main board temperature 1, 0.1℃ |
|  | 3 | INT16 | DriverTemperature2 |  | ro | Main board temperature 2, 0.1℃ |
|  | 4 | INT16 | MotorTemperature1 |  | ro | Motor temperature 1, 0.1℃ |
|  | 5 | INT16 | MotorTemperature2 |  | ro | Motor temperature 2, 0.1℃ |
|  | 6 | INT16 | MaxDriverTemperature |  | rw | Driver temperature threshold, 0.1℃, alarm and shutdown when the main board temperature exceeds the threshold |
|  | 7 | INT16 | MaxMotorTemperature |  | rw | Motor temperature threshold, 0.1℃, alarm and shutdown when the motor temperature exceeds the threshold |
| 0x6041 | 0 | UINT32 | Statusword | 0 | ro |  |
|  |  |  |  | 1 |  |  |
|  |  |  |  | 2 |  | Undervoltage |
|  |  |  |  | 3 |  | Overvoltage |
|  |  |  |  | 4 |  |  |
|  |  |  |  | 5 |  |  |
|  |  |  |  | 6 |  | Excessive phase C current |
|  |  |  |  | 7 |  | Excessive phase B current |
|  |  |  |  | 8 |  | Excessive phase A current |
|  |  |  |  | 9 |  | Phase C current offset calibration error |
|  |  |  |  | 10 |  | Phase B current offset calibration error |
|  |  |  |  | 11 |  | Phase A current offset calibration error |
|  |  |  |  | 12 |  | Motor temperature point 2 warning |
|  |  |  |  | 13 |  | Motor temperature point 1 warning |
|  |  |  |  | 14 |  | Driver board temperature point 2 warning |
|  |  |  |  | 15 |  | Driver board temperature point 1 warning |
|  |  |  |  | 16 |  | Motor temperature point 2 overhigh |
|  |  |  |  | 17 |  | Motor temperature point 1 overhigh |
|  |  |  |  | 18 |  | Driver board temperature point 2 overhigh |
|  |  |  |  | 19 |  | Driver board temperature point 1 overhigh |
|  |  |  |  | 20 |  |  |
|  |  |  |  | 21 |  |  |
|  |  |  |  | 22 |  | Position reverse over-limit |
|  |  |  |  | 23 |  | Position forward over-limit |
|  |  |  |  | 24 |  |  |
|  |  |  |  | 25 |  |  |
|  |  |  |  | 26 |  | Encoder fault |
|  |  |  |  | 27 |  | Encoder communication abnormality |
|  |  |  |  | 28 |  | Driver register reset |
|  |  |  |  | 29 |  | Driver fault |
|  |  |  |  | 30 |  | Driver communication abnormality |
|  |  |  |  | 31 |  | CAN FD PDO interrupt |
| 0x6042 | 0 | UINT16 | Statusword | 0 | ro | Encoder over-speed warning |
|  |  |  |  | 1 |  | Encoder temperature out of range warning |
|  |  |  |  | 2 |  | Encoder magnetic field too weak warning |
|  |  |  |  | 3 |  | Encoder magnetic field too strong warning |
|  |  |  |  | 4 |  | Encoder battery undervoltage warning (below 2.9V) |
|  |  |  |  | 5 |  | Encoder battery interrupt error (below 2.7V) |
|  |  |  |  | 6 |  | Encoder warning |
|  |  |  |  | 7 |  | Encoder error |
| 0x6043 | 0 | UINT16 | Statusword | 0 | ro | VDS overcurrent fault of C low-side MOSFET |
|  |  |  |  | 1 |  | VDS overcurrent fault of C high-side MOSFET |
|  |  |  |  | 2 |  | VDS overcurrent fault of B low-side MOSFET |
|  |  |  |  | 3 |  | VDS overcurrent fault of B high-side MOSFET |
|  |  |  |  | 4 |  | VDS overcurrent fault of A low-side MOSFET |
|  |  |  |  | 5 |  | VDS overcurrent fault of A high-side MOSFET |
|  |  |  |  | 6 |  | DRV overtemperature shutdown |
|  |  |  |  | 7 |  | Undervoltage lockout fault |
|  |  |  |  | 8 |  | Gate driver fault |
|  |  |  |  | 9 |  | VDS monitor overcurrent fault |
|  |  |  |  | 10 |  | Driver fault |
|  |  |  |  | 11 |  |  |
|  |  |  |  | 12 |  |  |
|  |  |  |  | 13 |  |  |
|  |  |  |  | 14 |  |  |
|  |  |  |  | 15 |  |  |
| 0x6044 | 0 | UINT16 | Statusword | 0 | ro | Gate driver fault of C low-side MOSFET |
|  |  |  |  | 1 |  | Gate driver fault of C high-side MOSFET |
|  |  |  |  | 2 |  | Gate driver fault of B low-side MOSFET |
|  |  |  |  | 3 |  | Gate driver fault of B high-side MOSFET |
|  |  |  |  | 4 |  | Gate driver fault of A low-side MOSFET |
|  |  |  |  | 5 |  | Gate driver fault of A high-side MOSFET |
|  |  |  |  | 6 |  | VCP charge pump and/or VGLS undervoltage fault |
|  |  |  |  | 7 |  | Overheat warning |
|  |  |  |  | 8 |  | Phase C sense amplifier overcurrent |
|  |  |  |  | 9 |  | Phase B sense amplifier overcurrent |
|  |  |  |  | 10 |  | Phase A sense amplifier overcurrent |
|  |  |  |  | 11 |  |  |
|  |  |  |  | 12 |  |  |
|  |  |  |  | 13 |  |  |
|  |  |  |  | 14 |  |  |
|  |  |  |  | 15 |  |  |
| 0x6060 | 0 | UINT8 | CtrMode | 0 | ro | 1-Position Mode, 2-Thrust Mode, 3-Speed Mode, 4-Current Mode, 5-Electric angle zero calibration, 6-Open-loop FOC |
| 0x607D | 0 | UINT8 | Software position limit | 2 | ro |  |
|  | 1 | INT16 | Min position limit |  | rw | Minimum position, unit: 10um |
|  | 2 | INT16 | Max position limit |  | rw | Maximum position, unit: 10um |

Appendix 2: PDO1 Status Bits

| Status bit Description | Status bit Description |
| --- | --- |
| bit0 | Reserved |
| bit1 | Overvoltage alarm |
| bit2 | Undervoltage alarm |
| bit3 | Overhigh internal motor temperature |
| bit4 | Overhigh controller temperature |
| bit5 | Motor stall alarm, 0X20 |
| bit6 | Position out of limit range, 0X40 |
| bit7 | Encoder communication abnormality, 0X80 |
| bit8 | Encoder fault |
| bit9 | Overcurrent alarm |
| bit10 | Driver communication abnormality |
| bit11 | Driver fault |
| bit12 | Driver abnormality |
| bit13 | - |
| bit14 | - |
| bit15 | - |

Appendix 3: PDO4 Fault Codes

| Error Code | Hex Value | Error Description | Compatible | Remarks |
| --- | --- | --- | --- | --- |
| 0 | 0x0000 | No error | - | - |
| - | 0x1xxx | Voltage error | - | - |
| 4097 | 0x1001 | Overvoltage | √ | Compatible |
| 4098 | 0x1002 | Undervoltage | √ | Compatible |
| - | 0x2xxx | Current error | - | - |
| 8193 | 0x2001 | Phase A current offset calibration error | √ | Compatible |
| 8209 | 0x2011 | Phase B current offset calibration error | √ | Compatible |
| 8225 | 0x2021 | Phase C current offset calibration error | √ | Compatible |
| 8195 | 0x2003 | Excessive phase A current | √ | Compatible |
| 8211 | 0x2013 | Excessive phase B current | √ | Compatible |
| 8227 | 0x2023 | Excessive phase C current | √ | Compatible |
| - | 0x3xxx | Temperature error | - | - |
| 12289 | 0x3001 | MOS temperature point 1 warning | √ | Derating strategy |
| 12305 | 0x3011 | MOS temperature point 2 warning | √ | Derating strategy |
| 12290 | 0x3002 | MOS temperature point 1 overhigh | √ | Stop output |
| 12306 | 0x3012 | MOS temperature point 2 overhigh | √ | Stop output |
| 12291 | 0x3003 | Motor temperature point 1 warning | √ | Derating strategy |
| 12307 | 0x3013 | Motor temperature point 2 warning | √ | Derating strategy |
| 12292 | 0x3004 | Motor temperature point 1 overhigh | √ | Stop output |
| 12308 | 0x3014 | Motor temperature point 2 overhigh | √ | Stop output |
| - | 0x4xxx | Reference value error | - | - |
| 16385 | 0x4001 | Position reference out of forward range | - | - |
| 16401 | 0x4011 | Position reference out of reverse range | - | - |
| - | 0x5xxx | Feedback value error | - | - |
| 20481 | 0x5001 | Position feedback out of forward range | - | - |
| 20497 | 0x5011 | Position feedback out of reverse range | - | - |
| - | 0x7xxx | Sensor error | - | - |
| 28673 | 0x7001 | Internal encoder connection lost | - | - |
| 28675 | 0x7003 | Internal encoder data error | - | - |
| - | 0x8xxx | Communication error | - | - |
| 32770 | 0x8002 | CAN FD PDO interrupt | √ | - |

Appendix 4: PDO4 Status Codes

| 0x20xx---Enter standby mode0x40xx---Enter servo mode0x41xx---Enter PVT mode0x4Fxx---Enter servo-off mode | bit 7: 1 - MOTOR ON, 2 - MOTOR OFF, indicating whether PWM is output; (Used for DriveStatus_DriveStatus, write 0 to DriveCtrl_DriveCtrl) bits 15:12 Motor mode 0 - 1 - 2 - READY，standby state; 3 - CFG，configuration; bits 11:8 Configuration item 0 - 1 - Save;； 2 - Zero calibration； 3 - 5 - Clear error； 4 - SERVO， 0 - Ready； 1 - Current loop （CSPVT）； 3 - Speed loop； 4 - Position loop； 5 - 6 - Encoder calibration; D - Torque sensor calibration; F - Servo-off; |
| --- | --- |